COMPUTER NETWORKS LABORATORY (21CSC302J)

Lab Record – Lab 1 and Lab 3

Lab 1: Introduction to Packet Tracer, Peer-to-Peer Communication, Study of Cables and its Color Codes

Lab 3: Router Configuration (Creating Passwords, Configuring Interfaces)

| Field | Details |
| --- | --- |
| Name | Kirthik Kasar P |
| Register Number | RA2411030050043 |
| Programme | B.Tech CSE (Cyber Security) |
| Course | Computer Networks (21CSC302J) |
| Tool Used | Cisco Packet Tracer |

# Lab 1: Introduction to Packet Tracer, Peer-to-Peer Communication, Study of Cables and its Color Codes

## Aim

To get familiar with Cisco Packet Tracer, build a simple two-PC network through a switch, assign IP addresses, and verify peer-to-peer communication using the ping command, and to study network cables and their colour codes.

## Requirements

- Cisco Packet Tracer

- 2 PCs (PC-PT), 1 switch (2960-24TT), copper straight-through cables

## Procedure

- Launch Cisco Packet Tracer and familiarise yourself with the workspace, device selection area and tools.

- Drag two computers (PC-PT) and one switch onto the workspace.

- Connect each computer to the switch using straight-through Ethernet cables.

- On PC0, open Desktop > IP Configuration and assign IP address 192.168.1.1 with subnet mask 255.255.255.0.

- On PC1, open Desktop > IP Configuration and assign IP address 192.168.1.2 with subnet mask 255.255.255.0.

- On PC0, open Desktop > Command Prompt and test connectivity with ping 192.168.1.2.

- Observe the replies to confirm that the two computers can communicate.

- Study the Ethernet cable types and note the wire colour codes.

## Network Topology

PC0 and PC1 are each connected to Switch0 (2960-24TT) with copper straight-through cables. The green link lights show that both links are up.

Figure 1.1: Topology – PC0 and PC1 connected through Switch0

## IP Configuration

| Device | IPv4 Address | Subnet Mask | Interface |
| --- | --- | --- | --- |
| PC0 | 192.168.1.1 | 255.255.255.0 | FastEthernet0 |
| PC1 | 192.168.1.2 | 255.255.255.0 | FastEthernet0 |

Figure 1.2: IP configuration of PC0 (192.168.1.1 / 255.255.255.0)

Figure 1.3: IP configuration of PC1 (192.168.1.2 / 255.255.255.0)

## Connectivity Test (Ping)

Command executed on PC0:

C:\>ping 192.168.1.2

Figure 1.4: Ping from PC0 to PC1 – 4 sent, 4 received, 0% loss

## Study of Cables and Colour Codes

Straight-through cable: used to connect unlike devices (e.g. computer to switch). Both ends use the same wiring standard (T568B).

| Pin | Colour (T568B) | Used for |
| --- | --- | --- |
| 1 | White/Orange | Transmit + |
| 2 | Orange | Transmit − |
| 3 | White/Green | Receive + |
| 4 | Blue | Unused |
| 5 | White/Blue | Unused |
| 6 | Green | Receive − |
| 7 | White/Brown | Unused |
| 8 | Brown | Unused |

Crossover cable: used to connect like devices (e.g. computer to computer). One end is wired T568A and the other T568B, so the transmit pair (pins 1, 2) on one side connects to the receive pair (pins 3, 6) on the other.

## Result

Two PCs were connected through a switch and configured with addresses in the 192.168.1.0/24 network. The ping from PC0 to PC1 (192.168.1.2) succeeded with 0% packet loss, confirming successful peer-to-peer communication. The cable types and colour codes were studied.

# Lab 3: Router Configuration (Creating Passwords, Configuring Interfaces)

## Aim

To build a simple network with a router, a switch and two computers, secure the router with console, enable and VTY passwords, assign IP addresses to the PCs, and verify connectivity.

## Requirements

- Cisco Packet Tracer

- 1 router (ISR4331), 1 switch (2960-24TT), 2 PCs (PC-PT), copper straight-through cables

## Procedure

- Open Packet Tracer and place a router, a switch and two PCs on the workspace.

- Connect the PCs to the switch and the switch to the router using straight-through Ethernet cables.

- Click the router and open the CLI tab.

- Enter privileged and global configuration mode using enable and configure terminal.

- Set the console password: line console 0, password cisco, login.

- Set the enable password: enable password cisco.

- Set the VTY password: line vty 0 4, password cisco, login.

- Configure the router interface with an IP address and enable it with no shutdown (as per the lab procedure: interface gig0/0, ip address 192.168.1.1 255.255.255.0).

- Assign IP addresses to the PCs with 192.168.1.1 as the default gateway.

- Use the ping command to test connectivity between the computers.

## Network Topology

Router0 (ISR4331) is connected to Switch0 (2960-24TT), and PC0 and PC1 are connected to the switch. All links are up (green indicators).

Figure 3.1: Topology – Router0, Switch0, PC0 and PC1

## Router Password Configuration

Commands entered in the router CLI:

Router>enable

Router#configure terminal

Router(config)#line console 0

Router(config-line)#password cisco

Router(config-line)#login

Router(config-line)#exit

Router(config)#enable password cisco

Router(config)#line vty 0 4

Router(config-line)#password cisco

Router(config-line)#login

Router(config-line)#exit

Figure 3.2: Router CLI – console, enable and VTY passwords configured

## IP Configuration of PCs

| Device | IPv4 Address | Subnet Mask | Default Gateway |
| --- | --- | --- | --- |
| PC0 | 192.168.1.2 | 255.255.255.0 | 192.168.1.1 |
| PC1 | 192.168.1.3 | 255.255.255.0 | 192.168.1.1 |

Figure 3.3: IP configuration of PC0 (192.168.1.2, gateway 192.168.1.1)

Figure 3.4: IP configuration of PC1 (192.168.1.3, gateway 192.168.1.1)

## Connectivity Test (Ping)

Command executed from PC0 (to PC1, 192.168.1.3) and, in the second screenshot, from PC1:

C:\>ping 192.168.1.3

Figure 3.5: Ping from PC0 to PC1 (192.168.1.3) – 4 sent, 4 received, 0% loss

Figure 3.6: Ping test run from PC1 – 4 sent, 4 received, 0% loss

## Result

The router was secured with console, enable and VTY passwords, the PCs were configured in the 192.168.1.0/24 network with 192.168.1.1 as the default gateway, and the ping from PC0 to PC1 (192.168.1.3) returned replies with 0% packet loss, confirming connectivity.